# 03 - Power Failure

Bridge Emergency Actions

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Master | CALLED |  |
| 02 | General Alarm if Imminent Danger | SOUNDED |  |
| 03 | Main Engines | REDUCED TO MINIMUM |  |
| 04 | Engine Room | INFORMED |  |
| 05 | Both Steering Pump | STARTED |  |
| 06 | Not Under Command Signals | EXHIBITED |  |
| 07 | MRCC and Nearby Vessels | ALERTED |  |

WARNING: If in danger of grounding or collision consider:

- Stop engines immediately

- Emergency anchoring

- Request towage assistance

Engine Room Emergency Actions

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 08 | Chief Engineer | CALLED |  |
| 09 | Bridge and Engine Room Communication | ESTABLISHED |  |
| 10 | Emergency Generator | STARTED |  |

Before Resuming Passage

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 11 | All Vessel Systems Functionality | CHECKED |  |
| 12 | Not Under Command to Navigation Signals | CHANGED |  |
| 13 | Navigation and Traffic in Vicinity | ASSESSED |  |

| Crew Member |  | Signature |
| --- | --- | --- |
| Full Name |  | I hereby confirm that I have checked and executed all the above. |
| Date and time |  |  |