# 04 - Grounding

Immediate Primary Actions

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Main Engines | STOPPED |  |
| 02 | Master | CALLED |  |
| 03 | General Alarm | SOUNDED |  |
| 04 | Watertight Doors | CLOSED |  |
| 05 | Guests and Crew | MUSTERED |  |
| 06 | Personal Injuries | ASSESSED |  |

Damage Assessment and Control

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 07 | External Damages | ASSESSED |  |
| 08 | Possible Leakage, Tank Soundings | CHECKED |  |
| 09 | Response Team Stand-by for Integrity | READY |  |
| 10 | Bilge Pumps | STARTED |  |
| 11 | Damage Control if Required | STARTED |  |
| 12 | Oil Pollution Prevention Measures | IF REQUIRED |  |

Communication and Position Recording

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 13 | Distress Message if Required | TRANSMITTED |  |
| 14 | DPA and Company for Assistance | CONTACTED |  |
| 15 | Exact Position and Heading on Chart | FIXED |  |
| 16 | Exact Grounding Time | RECORDED |  |

Environmental and Re-floating Assessment

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 17 | Sketch Soundings Around Vessel | COMPLETED |  |
| 18 | Tide Table for High and Low Water | CHECKED |  |
| 19 | Weather Forecast, Wind and Current | ASSESSED |  |
| 20 | Risk of Waves and Current | EVALUATED |  |
| 21 | Re-float Attempt Strategy | ESTIMATED |  |

Documentation

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 22 | Position On the Chart | FIXED |  |
| 23 | Time And Events | LOGGED |  |
| 24 | Grounding Emergency Response | LOGGED |  |
| 25 | DPA and Company for Assistance | CONTACTED |  |