# 06 - Collision

Collision Response Priorities:

- Offer assistance to other vessel if capable

- Minimise pollution from both vessels

- Consider vessel stability and withdrawal if embedded

Immediate Primary Actions

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Main Engines | STOPPED |  |
| 02 | Master | CALLED |  |
| 03 | Chief Engineer | CALLED |  |
| 04 | General Alarm | SOUNDED |  |
| 05 | Watertight Doors | CLOSED |  |
| 06 | Guests and Crew | MUSTERED |  |
| 07 | Response Team for Damage Control | READY |  |
| 08 | Electrical Power in Affected Area | ISOLATED |  |
| 09 | Location and Extent of Flooding | ESTABLISHED |  |
| 10 | Water Ingress Rate | ESTIMATED |  |
| 11 | Bilge Pump using OWS | STARTED |  |

Follow-up Emergency Actions

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 12 | Damage Control Plan | DEPLOYED |  |
| 13 | Flooding Effects on Stability | ESTIMATED |  |
| 14 | Pollution Prevention per SOPEP Manual | READY |  |
| 15 | Head Count | EXECUTED |  |
| 16 | Abandon Ship Procedure if Required | IMPLEMENTED |  |
| 17 | Survival Craft and LSA | READY |  |

Communication and Position Recording

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 18 | Communication with the Other Vessel | ESTABLISHED |  |
| 19 | MRCC and Nearby Vessels | ALERTED |  |
| 20 | DPA and Company for Assistance | CONTACTED |  |
| 21 | Position On the Chart | FIXED |  |
| 22 | Pictures and Movies | TAKEN |  |
| 23 | Time And Events | CONTINUOUS |  |