# 18 - Piracy

Suspected piracy ship detected

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Master | CALLED |  |
| 02 | Searchlights directed the suspected ship | ACTIVATED |  |
| 03 | Rescue Coordination Centre | ALERTED |  |
| 04 | Alarm and assume security positions | SOUNDED |  |
| 05 | Fire hoses | CHARGED |  |
| 06 | DPA to get further advice and assistance | CONTACTED |  |

Piracy attempt is imminent

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 07 | SSAS Alert Button | ACTIVATED |  |
| 08 | Rescue Coordination Centre of imminent attack | ALERTED |  |
| 09 | Aggressive steering manoeuvres | BEGUN |  |
| 10 | Maximum speed maintained if navigation allows | MAINTAINED |  |
| 11 | Other ships in the area of imminent attack | WARNED |  |
| 12 | Spotlights used to blind pirates | USED |  |
| 13 | Fire hoses aimed at the ship in an attempt to swamp ship | AIMED |  |
| 14 | Fire hoses aimed directly at persons attempting to board | AIMED |  |
| 15 | Fire axes used to cast off grappling hooks and poles | USED |  |

Pirates start to board the ship

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 16 | Alarm to alert crew members to seek citadel | SOUNDED |  |
| 17 | Headcount | EXECUTED |  |
| 18 | Control over navigation systems | MAINTAINED |  |

Pirates start to leave the ship

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 19 | Do not attempt to stop pirates |  |  |
| 20 | Do not prevent pirates from taking items |  |  |
| 21 | Stay in citadel until the bridge has given the signal for all clear |  |  |

Pirates have left the ship

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 22 | Following information | REPORTED |  |
| 23 | Headcount of crew members | EXECUTED |  |
| 24 | Thorough search of all compartments | CONDUCTED |  |
| 25 | Inventory of ship’s inventory | CONDUCTED |  |
| 26 | Company security officer | NOTIFIED |  |