# 19 - Helicopter Evacuation

DO

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | VHF on Ch 16 to communicate with the helicopter | SET |  |
| 02 | if rescue area is aft | WIND 30° PORT BOW |  |
| 03 | if rescue area is forward | WIND 30° STARBOARD BOW |  |
| 04 | All deck area from loose gear | CLEARED |  |
| 05 | Rescue area | CLEARED |  |
| 06 | Flag to indicate wind direction | FLYING |  |
| 07 | Rescue area at night | ILLUMINATED |  |
| 08 | Rubber gloves to handle winch wire | READY |  |

DON’T

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 09 | Fire rockets or use line-throwing appliances in vicinity of helicopter | DO NOT |  |
| 10 | Illuminate the helicopter | DO NOT |  |
| 11 | Touch the winch man, winch hook until it has been earthed | DO NOT |  |
| 12 | Secure any lines passed down from helicopter | DO NOT |  |