# 07 - Pre-Arriving in Port

Pre-arrival procedures for port approach, including communication with port authorities, engine and deck configuration, and bridge equipment setup to ensure safe and compliant arrival in port.

Pre-Arrival Communication and Coordination

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | ETA Notice to ER and deck team | NOTIFIED |  |
| 02 | Port Authorities | CONTACTED |  |
| 03 | Pilot boarding time | CONFIRMED |  |
| 04 | Pre-Arrival Documentation | PREPARED |  |
| 05 | VHF Channel on VTS/Port | SET |  |

Engine and Machinery Configuration for Arrival

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 06 | Engine room ready for manoeuvre | CONFIRMED |  |
| 07 | Any defect from the E.R. | REPORTED |  |
| 08 | Main Engines to slow ahead | SET |  |
| 09 | Both Generators | STARTED |  |
| 10 | Bow thruster | READY |  |

Deck Configuration for Arrival

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 11 | Courtesy Flag | UP |  |
| 12 | Bridge wings | OPENED & SECURED |  |
| 13 | Anchor in sea mode | SECURED |  |
| 14 | Fenders | OUT |  |
| 15 | Mooring lines ready for use | READY |  |

Bridge Equipment Configuration for Arrival

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 16 | Gyro/mag compass | CHECKED |  |
| 17 | ECDIS on appropriate scale | SET |  |
| 18 | Both Generators | STARTED |  |
| 19 | Bow thruster | ON & TESTED |  |
| 20 | Steering Pumps | BOTH STARTED |  |
| 21 | Manual Steering | TESTED |  |
| 22 | Both Radars | STAND-BY |  |