# 14 - GMDSS Testing

Systematic testing schedule for Global Maritime Distress and Safety System equipment ensuring regulatory compliance and operational readiness per SOLAS Chapter IV requirements.

Daily Equipment Tests

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Battery Voltage Above 26V | CHECKED |  |
| 02 | VHF Fixed Station Transmit Test | COMPLETED |  |
| 03 | VHF Transmit Red Light Indication | CHECKED |  |
| 04 | Battery Voltage During Transmission | CHECKED |  |
| 05 | VHF Channel 16 Voice Reception | CONFIRMED |  |

Weekly Equipment Tests

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 06 | Voice Transmission for Radio Check | COMPLETED |  |
| 07 | VHF DSC Call to Self MMSI Number | TESTED |  |
| 08 | Tender to Fixed Station Transmission | TESTED |  |
| 09 | Portable VHF Battery Check | CHECKED |  |
| 10 | Non-Rechargeable Battery | CHECKED |  |

Monthly Equipment Tests

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 11 | Tender VHF Transmission at Sea | TESTED |  |
| 12 | Tender Battery Voltage Above 14V | CHECKED |  |

Annual Equipment Tests

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 13 | EPIRB Battery Condition | CHECKED |  |
| 14 | EPIRB Hydrostatic Release Unit | CHECKED |  |
| 15 | EPIRB Self-Test Function | COMPLETED |  |
| 16 | SART Test Against X-Band Radar | CHECKED |  |

GMDSS Logbook

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 17 | GMDSS Test Results Recorded in Log Book | LOGGED |  |
| 18 | Equipment Testing | COMPLETED |  |

| Crew Member |  | Signature |
| --- | --- | --- |
| Full Name |  | I hereby confirm that I have checked and executed all the above. |
| Date and time |  |  |