# 08 - Bunkering Preparation

Planning and Communication

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Bunker Loading Plan | AGREED |  |
| 02 | Duty Bridge Officer | ADVISED |  |
| 03 | Red Flag Display | RAISED |  |
| 04 | Vessel Stability and Moorings | VERIFIED |  |
| 05 | Ship, Shore and Barge Communication | TESTED |  |
| 06 | No Smoking Signs | DISPLAYED |  |

Safety and Environmental Preparation

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 07 | Foam Extinguishers at Bunker Station | READY |  |
| 08 | Scuppers on Bunkering Side | PLUGGED |  |
| 09 | Opposite Bunker Station | BLANKED |  |
| 10 | SOPEP Kit at Bunkering Station | READY |  |

Tank and System Configuration

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 11 | Fuel Tank High Level Alarm to 90% | SET |  |
| 12 | Overflow Tank | EMPTY |  |
| 13 | Overflow Alarm | TESTED |  |
| 14 | All Fuel Tank Levels | LOGGED |  |
| 15 | All Remote-Control Valves | TESTED |  |
| 16 | All Valves in Correct Position | CHECKED |  |
| 17 | Fuel Separator Link to Main Manifold | CLOSED |  |
| 18 | Vessel Fuel Meter | RESET |  |
| 19 | Biocide System | FILLED |  |

Equipment and Documentation Verification

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 20 | Filling Hose and Connection Integrity | CHECKED |  |
| 21 | Filling Hose | SECURED |  |
| 22 | Bunkering Delivery Note | AVAILABLE |  |
| 23 | Quality and Quantity Fuel Agreement | CHECKED |  |
| 24 | Pumping Rate | AGREED |  |
| 25 | Supplier Emergency Stop Procedure | AGREED |  |
| 26 | Supplier Fuel Meter | CHECKED |  |
| 27 | Supplier Fuel Tanks | INSPECTED |  |
| 28 | Bunkering Team Members | BRIEFED |  |