# 09 - Bunkering Operations

Active monitoring and control procedures during fuel transfer operations ensuring safe pumping rates, proper distribution, and continuous system surveillance.

System Operation and Flow Control

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Biocide System Running and Pump Rate | SET |  |
| 02 | Pumping Initiated at Minimum Rate | AGREED |  |
| 03 | Distribution to Correct Tank | VERIFIED |  |
| 04 | Closed Tank Levels Steady | VERIFIED |  |
| 05 | Pump Rate Adjusted to Maximum Rate | SET |  |
| 06 | Minimum Rate When Tank Isolation | IMPLEMENTED |  |

Continuous Monitoring Requirements

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 07 | Communication and Deck Watch | MAINTAINED |  |
| 08 | Tank Levels at All Times | MONITORED |  |
| 09 | Overflow Side Glass | CHECKED |  |
| 10 | Trim and List for Even Keel | MONITORED |  |

System Securing and Disconnection

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 11 | Hose Drained and Blown Through | COMPLETED |  |
| 12 | Filling Valves | CLOSED |  |
| 13 | All Remote Controlled Valves | CLOSED |  |
| 14 | Filling Hose | DISCONNECTED |  |
| 15 | Bunker Station | BLANKED |  |

Verification and Documentation

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 16 | Supplier Meter Versus Tank Level | COMPARED |  |
| 17 | Fuel Samples | OBTAINED |  |
| 18 | All Receipts and Certificates | RECEIVED |  |
| 19 | Vessel Load Line | CHECKED |  |

Final Actions and Records

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 20 | Completion | BRIDGE INFORMED |  |
| 21 | Red Bravo Flag | LOWERED |  |
| 22 | Bunkering in Oil Record Book | LOGGED |  |
| 23 | High and Low Level Alarms | RESET |  |

| Crew Member |  | Signature |
| --- | --- | --- |
| Full Name |  | I hereby confirm that I have checked and executed all the above. |
| Date and time |  |  |