# 10 - Dry Dock Entry

Comprehensive preparation for dry dock entry including pre-operation verification, critical systems shutdown, and power transition from main generators to emergency power.

Pre-Operation Verification

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Arrival Checklist | COMPLETED |  |
| 02 | Tank Contents | LOGGED |  |
| 03 | Wastewater Tanks at Minimum Level | MAINTAINED |  |
| 04 | Vessel at Even Keel, Trim and List Zero | VERIFIED |  |

Initial Systems Shutdown After Keel on Dock

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 05 | ICCP and Cathodic Protection | OFF |  |
| 06 | Hamann System | OFF |  |
| 07 | Chilled Water System | OFF |  |
| 08 | Shaft Cooling Pumps | OFF |  |
| 09 | Emergency Generator | READY |  |
| 10 | Non-Essential Consumer Load | MINIMISED |  |

Power Transition to Emergency Generator

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 11 | Generator Load to Single Unit | SET |  |
| 12 | Generators to Manual Mode | SET |  |
| 13 | Emergency Generator | ONLINE |  |
| 14 | Online Generator Circuit Breaker | OPEN |  |
| 15 | Main Generator | OFF |  |
| 16 | Central Cooling Sea Water Pumps Manual | OFF |  |
| 17 | Central Cooling Generator Pumps Manual | OFF |  |
| 18 | Stabilisers Manual Pin | LOCKED |  |

Shore Power Connection Preparation

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 19 | Emergency Switchboard Link | ON |  |
| 20 | Shore Cooling Water | CONNECTED |  |
| 21 | Shore Power Cable | CONNECTED |  |
| 22 | Air Conditioning Sea Water Pump Breakers | OFF |  |
| 23 | Shore Power Breakers | ON |  |
| 24 | Shore Power Push Button | ON |  |
| 25 | Emergency Generator Link Key | NORMAL |  |

| Crew Member |  | Signature |
| --- | --- | --- |
| Full Name |  | I hereby confirm that I have checked and executed all the above. |
| Date and time |  |  |