# 12 - Undocking

Complete systematic vessel launch procedure ensuring all machinery, safety systems, and sea connections are properly configured for safe water entry and initial operations. This checklist covers pre-launch preparations through post-launch verification.

Pre-Launch Shore Operations

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 01 | Briefing With Shipyard And Captain | COMPLETED |  |
| 02 | Machinery Breakers | TAGGED OFF |  |
| 03 | Main Engine And Gearbox Cooling Systems | FILLED |  |
| 04 | All Seacock Valves | CLOSED |  |
| 05 | Air Conditioning Seawater Circuit | FILLED/VENTED |  |
| 06 | Emergency Generator Mode | TO MANUAL |  |
| 07 | All Bilge Wells | DRY/ALARMS CLEAR |  |

Shore Power And Utilities Disconnection

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 08 | Wastewater Shore Discharge Valve | CLOSED |  |
| 09 | Wastewater Hose | DISCONNECTED |  |
| 10 | Fresh Water Hose | DISCONNECTED |  |
| 11 | Air Conditioning Units | OFF |  |
| 12 | Air Conditioning Shore Cooling | DISCONNECTED |  |
| 13 | Shore Power | SWITCHED OFF |  |
| 14 | Shore Power Cable | DISCONNECTED |  |

Vessel In Water - Systems Commissioning

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 15 | Seawater Valves Integrity | CHECKED |  |
| 16 | Main Sea Chest Valves | OPENED |  |
| 17 | Forward Sea Chest Valves | OPENED |  |
| 18 | Central Plate Heat Exchanger | VENTED |  |
| 19 | Seawater Discharge Valves | OPENED |  |
| 20 | Central Cooling Generator Pumps | ON |  |
| 21 | Power Management System | READY |  |
| 22 | Primary Generator | STARTED |  |
| 23 | Main Switchboard Generator | ONLINE |  |
| 24 | Central Cooling Seawater Pump | SET TO AUTO |  |

Main Systems Verification

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 25 | All Circuit Breakers | SWITCHED ON |  |
| 26 | Seawater Pressure Parameters | WITHIN LIMITS |  |
| 27 | Remaining Generators | STARTED |  |
| 28 | Fire And Bilge Pumps | PRIMED/TESTED |  |
| 29 | Shaft Cooling Pumps | PRIMED/TESTED |  |
| 30 | Propeller Shaft Seals | CHECKED |  |
| 31 | Air Conditioning Units | TESTED |  |
| 32 | Main Engine Controls | LOCAL |  |
| 33 | Main Engine Cooling System | SATISFIED |  |
| 34 | Stabilisers Lock Pin | UNLOCK |  |

Final Systems Check

| # | ACTION | VERB |  |
| --- | --- | --- | --- |
| 35 | Bow Thruster Seals | CHECKED |  |
| 36 | Propeller Shaft Oil Tanks | CHECKED |  |
| 37 | Fin Stabiliser Shaft Seals | CHECKED |  |
| 38 | Steering Gear Shaft Seals | CHECKED |  |

| Crew Member |  | Signature |
| --- | --- | --- |
| Full Name |  | I hereby confirm that I have checked and executed all the above. |
| Date and time |  |  |